| Robot type |  |  |  | MZ25-01 |
| --- | --- | --- | --- | --- |
| Structure |  |  |  | Articulated robot |
| Degree of Freedom |  |  |  | 6 |
| Maximum motion range | Arm | J1 | Swivel | ±2.97rad (±170°) |
|  |  | J2 | Forward/ Backward | +1.83 to -2.62rad (+105 to -150°) |
|  |  | J3 | Upward/ Downward | +5.04 to -2.81rad (+289 to -161°) |
|  | Wrist | J4 | Rotation2 | ±3.32rad (±190°) |
|  |  | J5 | Bend | ±2.53rad (±145°) |
|  |  | J6 | Rotation1 | ±6.28rad (±360°) |
| Maximum Velocity*1 | Arm | J1 | Swivel | 3.67rad/s (210°/s) |
|  |  | J2 | Forward/ Backward | 3.23rad/s (185°/s) |
|  |  | J3 | Upward/ Downward | 4.71rad/s (270°/s) |
|  | Wrist | J4 | Rotation2 | 7.33rad/s (420°/s) |
|  |  | J5 | Bend | 7.33rad/s (420°/s) |
|  |  | J6 | Rotation1 | 11.73rad/s (672°/s) |
| Maximum Payload |  |  | Wrist | 25kg |
| Maximum static load torque |  | J4 | Rotation2 | 52N·m |
|  |  | J5 | Bend | 52N·m |
|  |  | J6 | Rotation1 | 32N·m |
| Maximum moment of inertia*2 |  | J4 | Rotation2 | 2.4kg·m2 |
|  |  | J5 | Bend | 2.4kg·m2 |
|  |  | J6 | Rotation1 | 1.3kg·m2 |
| Maximum Reach |  |  |  | 1,882mm |
| Position repeatability*3 |  |  |  | ±0.05mm |
| Air tubes |  |  |  | IN : Ø6×1, OUT : Ø10×1 |
| Application cables |  |  |  | 24 wires (Including wires for optional valves) |
| Mounting Condition |  |  |  | Floor / Inverted / Angle |
| Ambient Temperature |  |  |  | 0 to 45C°*4 |
| Ambient Humidity |  |  |  | 20 to 85%RH (No dew or frost allowed) |
| Protection class |  |  |  | IP 67 equivalent (dust-proof, moisture-resistant) |
| Robot weight |  |  |  | 250kg |